Runbook: No Internet / Slow Internet

Audience: Level 1 Help Desk / Desktop Support
Goal: Restore connectivity or escalate with clear evidence.

Symptoms (What the user reports / what you observe)

Websites time out or load extremely slowly

Wi-Fi/Ethernet shows 'Connected' but no browsing

VPN disconnects frequently (may be symptom, not root cause)

Multiple users in the same area report similar issues (scope indicator)

Likely Causes (most common first)

User device is not actually connected (Wi-Fi disconnected, cable loose, airplane mode)

Local network issue (bad cable/port, weak Wi-Fi signal, AP issue)

DHCP failure (no valid lease; APIPA 169.254.x.x)

DNS resolution issue (IP works, names fail)

Captive portal / network access control block

Upstream outage (router/ISP) or broader network incident

Resolution Steps (Level 1)

1) Confirm scope (fast)

Ask: 'Is it only you, or are others nearby affected?'

If multiple users are affected, note location and time, then jump to Escalation Criteria.

2) Physical / connection check

Ethernet: verify cable seated, link light on NIC/switch port; try a known-good cable/port if available.

Wi-Fi: verify correct SSID; toggle Wi-Fi off/on; move closer to AP if signal is weak.

Confirm device is not in airplane mode.

3) Capture network state (Windows)

Run: ipconfig /all

Record IPv4 address, default gateway, DNS servers, and whether DHCP is enabled.

4) If APIPA (169.254.x.x) or no default gateway (DHCP suspect)

Attempt lease renewal: ipconfig /release then ipconfig /renew

If renewal fails, restart the network adapter and retry; if others are impacted, escalate.

5) Connectivity isolation tests (no DNS first)

Test external IP: ping 8.8.8.8

Fail: likely routing/local network issue; continue to gateway checks.

Success: IP path works; proceed to DNS test.

6) DNS test (name resolution)

Run: ping google.com

If IP works but names fail: flush DNS (ipconfig /flushdns) and retest.

Optional if still failing: netsh winsock reset (reboot required).

7) Gateway and local path checks (if external IP ping fails)

Ping default gateway: ping <default_gateway_ip>

Gateway fail: local link/Wi-Fi/AP/switch issue; try cable/port, reconnect Wi-Fi, adapter reset.

Gateway success but external fail: upstream routing/ISP/AP issue; document and escalate.

8) Captive portal / NAC quick check (if on guest/corp Wi-Fi)

Open a browser to trigger captive portal if policy allows; authenticate and retest.

Validation (confirm the fix)

User can open 2-3 different websites

ping 8.8.8.8 and ping google.com succeed

If relevant: VPN reconnects and stays stable for several minutes

Evidence to record in the ticket

Scope: single user vs multiple users

Output snippets: ipconfig /all, ping results

Exact error messages and timestamps

Location (floor/room) and connection type (Wi-Fi SSID or Ethernet)

Escalation Criteria (stop Level 1 here)

Multiple users are affected (suspected AP/switch/router/ISP incident)

DHCP renewal consistently fails (especially across multiple devices)

Gateway unreachable after basic physical/Wi-Fi checks

DNS appears broken for multiple users or across a site

Suspected security/NAC block or policy enforcement issue

Last updated: 2026-02-18